GLOBAL SIP TRUNK PROVIDER\nRESEARCH & ASTERISK INTEGRATION GUIDE

India + International carriers • SIP authentication • UDP/TCP/TLS • PJSIP

Prepared: 1 October 2026\nPurpose: shortlist PSTN connectivity providers and normalize their onboarding/configuration requirements for Asterisk.

# 1. Executive summary

Asterisk can interconnect with most standards-based SIP trunks. The main differences are not usually “Asterisk compatibility”; they are the provider’s product type, authentication model, permitted source/destination IPs, number/caller-ID rules, transport/security requirements, and commercial/regulatory conditions.

- Registration trunk: Asterisk sends REGISTER to a registrar using server_uri, client_uri, and usually outbound_auth. Registration state and SIP authentication are separate from the endpoint/AOR used to place calls.

- IP-authenticated trunk: no REGISTER is normally needed. The provider allowlists your public signaling IP; Asterisk sends INVITEs to the provider’s termination URI. Incoming traffic is matched to an endpoint by provider source IP using a PJSIP identify object.

- Credential-authenticated trunk: Asterisk sends an INVITE and responds to a 401/407 challenge using SIP Digest credentials. Some providers allow IP ACL and credentials together.

- Programmable voice API: if the service is REST/webhook-first and does not expose a standard SIP trunk or SIP interconnect, it is not a drop-in carrier trunk.

- UDP is common for SIP signaling, but not universal. TCP and TLS are also used; media is commonly RTP/UDP, while secure trunks may require SRTP.

Research limitation: this is a broad, source-linked survey, not literally every carrier worldwide. Product availability, onboarding fields, country coverage, and protocols can vary by region, account type, and contract. Where a provider’s public documentation does not disclose exact trunk fields, the report marks them “confirm with sales/engineering” rather than guessing.

# 2. Provider comparison matrix

| Provider / service | Product type | Likely required fields | Protocol / auth model | Asterisk fit | Official portal / docs | Important qualification |
| --- | --- | --- | --- | --- | --- | --- |
| BSNL Wings / NGN portal (India) | Internet telephony / SIP-client service; do not assume enterprise carrier trunk | Username + password + regional domain/proxy after activation; number assigned to subscriber | SIP registration; official guide gives regional domain and outbound proxy examples. Exact transport should be confirmed for the specific service. | Conditional / verify terms and access | https://ngn.bsnl.co.in/ ; https://ngn.bsnl.co.in/index.php?r=site/confighelp | Public documentation describes softphone setup. Confirm whether Asterisk/PBX use and automated calling are permitted for your subscription. |
| BSNL Enterprise SIP Trunk | Enterprise SIP trunk | Enterprise provisioning; service address, IP-PBX/SBC, public IP/network and number plan; exact auth/peer details supplied by circle | SIP/RTP; some BSNL documents discuss SIP trunk interconnection and security requirements; circle-specific design applies | Likely feasible after carrier engineering approval | https://eb.bsnl.co.in/ ; https://www.calcutta.bsnl.co.in/forms/EB_Form_SIP.pdf | Obtain written details for registration vs IP peering, signaling IPs, transport, codec, DID, concurrency, and allowed use. Terms restrict resale/call transit absent authorization. |
| JioBusiness SIP Trunk (India) | Enterprise IP-PBX SIP trunk | Sales-led provisioning; expect trunk peer details, customer IP/site, DIDs, channel count and routing profile. Public page does not list a universal credential form. | IP-based SIP trunk; UDP/TCP/TLS and authentication mode must be confirmed with Jio for the ordered service | Likely feasible; confirm exact interconnect profile | https://www.jio.com/business/services/voice-and-collaboration/sip-trunk/ | Jio advertises inbound/outbound, multiple DIDs and 10–5,000 sessions. Public marketing page does not establish exact PJSIP values. |
| Airtel Business SIP Trunk / SIP PRI (India) | Business trunk product; page uses SIP PRI terminology | Sales-led. Ask for signaling peer/registrar, customer public IP, pilot/DID numbers, channels, caller-ID policy | SIP/IP-PBX compatibility is described generally; specific UDP/TCP/TLS and authentication details need confirmation | Likely feasible if delivered as SIP over IP; confirm product architecture | https://www.airtel.in/b2b/sip-trunk | Public page is sales-oriented and does not publish a universal Asterisk configuration. Distinguish a SIP/IP service from a legacy PRI handoff. |
| Tata Tele Business Services (India) | Potential enterprise SIP/voice connectivity; exact current product and access model must be confirmed | Request product datasheet and handoff specification: SIP peer/FQDN, public IP, auth, DID, channels, codecs, transport | Not verified from the public materials reviewed for this report | Unknown until technical response | https://www.tatatelebusiness.com/ | Do not assume all Tata voice products are SIP trunks. Ask specifically for SIP trunk to customer-owned Asterisk/PJSIP IP-PBX. |
| Twilio Elastic SIP Trunking | Cloud PSTN SIP trunk | Termination SIP URI; IP ACL and/or Credential List; associate Twilio numbers for inbound origination; caller ID must be permitted | UDP default for origination toward customer; TCP/TLS options; secure trunking uses TLS + SRTP | Documented Asterisk interconnection guide | https://www.twilio.com/docs/sip-trunking ; https://www.twilio.com/docs/sip-trunking/sample-configuration | Twilio explicitly says not to REGISTER for this trunk. Use E.164 destinations including +. |
| Plivo Zentrunk | Cloud PSTN SIP trunk | Create inbound/outbound trunk; termination domain; IP ACL and/or username/password; attach numbers; outbound voice/caller-ID policies | Signaling 5060 UDP/TCP; 5061 TLS; media ports published by Plivo; IP ACL or digest credentials | Documented Asterisk guide | https://docs.plivo.com/docs/sip-trunking ; https://docs.plivo.com/docs/sip-trunking/interconnection-guides/asterisk | Plivo publishes Asterisk setup and technical specifications. Verify number/caller-ID rules for target country. |
| Telnyx | Cloud SIP trunk / SIP connection | Choose Credentials, IP Address, or FQDN connection; assign number and outbound voice profile; configure connection/region | UDP/TCP 5060; TLS 5061; RTP/UDP range per current connection documentation; TLS/SRTP available | Asterisk-compatible; provider publishes PBX guidance | https://telnyx.com/resources/how-to-setup-sip-trunk ; https://sip.telnyx.com/ | IP auth is appropriate for stable public IP; credentials for endpoints without stable IP. Regional SIP hostnames can differ. |
| Vonage SIP Trunking | Cloud SIP trunk | Trunk domain/endpoint; Digest username/password and/or IP ACL; add numbers and set SIP URIs | SIP trunk supports Digest authentication and ACL; TLS is advertised; confirm specific transport/port and media profile for region | Asterisk setup flow is documented | https://developer.vonage.com/en/sip/technical-details ; https://developer.vonage.com/en/sip/sip-dashboard | Do not confuse SIP Trunking with Vonage Voice API. SIP trunk credentials are not necessarily the account API key/secret. |
| Bandwidth | Carrier/SIP trunk offering; availability and onboarding vary by market/customer | Carrier-provisioned IP/SBC details, account/customer identifiers, DIDs, caller-ID and routing rules; request current interop guide | Confirm current product transport and authentication profile directly | Potentially compatible; verify product/region | https://www.bandwidth.com/ | Publicly available details reviewed here were insufficient to specify universal PJSIP fields; treat as sales-engineered carrier onboarding. |
| DIDWW | International DIDs and SIP trunking | Account, SIP trunk/termination details, IP/credentials depending on configured interconnection, purchased/ported numbers | Confirm transport, IP ranges and supported codecs in current technical portal for the selected product | Potentially compatible; verify product-specific settings | https://www.didww.com/ | Ask for the current Asterisk/PJSIP interconnection guide and the correct signaling/media IP ranges for your region. |

# 3. Provider-specific notes

## India: BSNL, Jio, Airtel and Tata

BSNL has distinct offerings that should not be conflated. Wings is documented as an app/SIP-client internet-telephony service, with subscriber activation and regional SIP domain/proxy details. Separately, BSNL Enterprise Business publishes SIP trunk application/security material. A consumer Wings credential should not be presumed to authorize a PBX trunk or unattended outbound calling. Confirm the applicable terms and technical interface with BSNL.

JioBusiness publicly describes its SIP trunk as IP-based connectivity for IP-PBXs with inbound/outbound calling and 10–5,000 sessions. Airtel Business markets SIP trunk/SIP PRI for business call distribution and DID. Their public pages are sales pages rather than complete protocol datasheets; request exact peer IPs/FQDNs, transport, registration/authentication method, RTP range, codec list, caller-ID format, and NAT/SBC requirements before implementation.

Tata Tele Business Services should remain “pending technical verification” in the provider registry until Tata confirms the precise product, supported customer-side SIP interface, and current integration guide. A business voice product or managed PBX is not automatically a SIP trunk terminating directly on a customer-owned Asterisk server.

## Global providers

Twilio is a clear example of a non-registration trunk: configure a Termination SIP URI plus at least one authentication method (IP ACL and/or credentials), associate inbound numbers with the trunk, and configure your Asterisk server as the origination destination. Twilio states UDP is the default for originating calls to customer infrastructure, with TCP/TLS options.

Plivo Zentrunk publishes a dedicated Asterisk guide and supports IP ACLs, credentials, or both. Its published signaling ports are 5060 for UDP/TCP and 5061 for TLS; check the current media range and caller-ID restrictions in the technical specification before opening firewall rules.

Telnyx offers credential, IP-address, and FQDN connection modes. Vonage SIP Trunking supports a dedicated SIP endpoint and Digest authentication and/or IP ACLs. Bandwidth and DIDWW are plausible carrier candidates, but their current service-specific interconnection parameters must be confirmed rather than copied from a generic guide.

# 4. What the fields mean

| Field | Meaning | When needed |
| --- | --- | --- |
| SIP registrar / server URI | Where Asterisk sends REGISTER requests. Example shape: sip:registrar.example.net | Registration-based service only |
| Client URI / auth username | Identity used in REGISTER and/or Digest authentication; may differ from the public phone number | Registration and/or credential-auth services |
| Password / secret | SIP Digest password, not necessarily the portal password or REST API secret | Credential authentication |
| Termination URI / outbound proxy | Provider destination to which Asterisk sends outbound INVITEs | Outbound trunk; may be FQDN or IP |
| Provider IP allowlist | Your public source IP added to the provider ACL; provider may also give IPs for inbound matching | IP-authenticated trunks |
| Origination URI / inbound destination | Your public Asterisk/SBC address where provider sends inbound calls | Inbound trunk |
| DID / pilot / phone number | Number(s) assigned or ported to the account and mapped to the trunk | Inbound; often caller ID for outbound |
| Caller ID / CLI policy | Which calling number formats and numbers are permitted | Outbound calls; country/product-specific |
| Transport | UDP, TCP or TLS for SIP signaling | All SIP interconnects |
| Media / RTP ports | UDP port range used for audio; SRTP may be required for secure trunking | All RTP-based calls |
| Codecs | Commonly PCMU/ulaw and PCMA/alaw; others depend on provider | Must have compatible codec on both ends |
| Concurrency / CPS | Maximum simultaneous calls and calls per second | Capacity planning and rate limiting |
| Failover URI / IPs | Secondary provider edge or destination | Resilience / high availability |

# 5. SIP transport: UDP, TCP, TLS and media

| Protocol | Typical use | Asterisk implementation notes |
| --- | --- | --- |
| SIP over UDP | Common signaling default; low overhead | Often port 5060. Must handle NAT mappings and retransmissions; allow provider signaling IPs. |
| SIP over TCP | Connection-oriented signaling; sometimes used by providers | Often port 5060. Keep connection alive where required; check provider-specific support. |
| SIP over TLS | Encrypted SIP signaling | Often port 5061. Configure a TLS transport and valid certificates/trust settings as required. |
| RTP over UDP | Real-time audio media; separate from SIP signaling | Open configured Asterisk RTP range in both directions and allow the provider media networks. |
| SRTP | Encrypted media | Must be negotiated and supported on both ends; TLS alone does not encrypt RTP media. |
| SIP REGISTER | Creates/renews a binding between an account/number and reachable contact | Not used by many IP-authenticated trunks, including Twilio Elastic SIP Trunking. |
| SIP INVITE + Digest challenge | Call setup; provider may challenge with 401/407 | Asterisk responds using outbound_auth credentials. |
| IP ACL / source-IP matching | Trust based on known public IP addresses | Requires stable public IP and careful inbound endpoint matching/firewall policy. |

Do not assume UDP just because a provider uses SIP. Confirm signaling transport, port, media range, SRTP/TLS requirements, and the provider’s full signaling/media IP list. SIP signaling and RTP media are separate flows.

# 6. Asterisk PJSIP configuration patterns

These are templates, not production-ready provider configs. Replace every example value and follow the provider’s exact domain, IP, transport, auth, caller-ID, and number-format requirements. Use chan_pjsip on current Asterisk releases unless you have a legacy reason to use chan_sip.

## Pattern A — IP-authenticated trunk (no registration)

; pjsip.conf — illustrative only
[provider-transport]
type=transport
protocol=udp
bind=0.0.0.0:5060

[carrier-aor]
type=aor
contact=sip:TERMINATION_HOST_FROM_PROVIDER

[carrier-endpoint]
type=endpoint
transport=provider-transport
aors=carrier-aor
context=from-carrier
disallow=all
allow=ulaw,alaw
direct_media=no
; Add provider-specific outbound_auth only if credentials are also required.

[carrier-identify]
type=identify
endpoint=carrier-endpoint
match=PROVIDER_SIGNALING_IP_OR_CIDR

Outbound dial example: Dial(PJSIP/${EXTEN}@carrier-endpoint). For an IP-authenticated provider, whitelist the actual static public IP used by Asterisk at the provider and configure all published provider signaling IPs for inbound matching. Do not match broad/untrusted networks.

## Pattern B — Credential-authenticated trunk

[carrier-auth]
type=auth
auth_type=userpass
username=PROVIDER_SIP_USERNAME
password=PROVIDER_SIP_PASSWORD

[carrier-aor]
type=aor
contact=sip:TERMINATION_HOST_FROM_PROVIDER

[carrier-endpoint]
type=endpoint
aors=carrier-aor
outbound_auth=carrier-auth
context=from-carrier
disallow=all
allow=ulaw,alaw
direct_media=no

A credential-authenticated trunk may not require REGISTER. The provider can challenge the outbound INVITE. Whether inbound calls use IP identification, a registration association, or another mechanism depends on the service.

## Pattern C — Registration-based service

[carrier-auth]
type=auth
auth_type=userpass
username=PROVIDER_USERNAME
password=PROVIDER_PASSWORD

[carrier-registration]
type=registration
outbound_auth=carrier-auth
server_uri=sip:REGISTRAR_HOST_FROM_PROVIDER
client_uri=sip:PROVIDER_ACCOUNT_OR_NUMBER@PROVIDER_DOMAIN
contact_user=INBOUND_ROUTE_USER
retry_interval=60

; Configure a separate endpoint and AOR for actual call routing.
; Registration alone does not fully configure inbound/outbound calling.

Asterisk documentation states outbound registration is separate from the endpoint/AOR configuration. The provider may require a specific client URI, contact user, outbound proxy, or transport parameter.

# 7. Which providers are not fit for Asterisk?

Based on the public material reviewed, do not label the mainstream SIP-trunk products from Twilio or Plivo as incompatible: both publish Asterisk interconnection guidance. Telnyx and Vonage also document standard SIP-trunk models suitable for SIP infrastructure. The more useful classification is “verified standard SIP”, “likely SIP but provider details pending”, or “not a drop-in trunk”.

| Classification | Services in this survey | Decision rule |
| --- | --- | --- |
| Verified standard SIP / documented Asterisk path | Twilio Elastic SIP Trunking; Plivo Zentrunk | Proceed to lab test with the provider’s current guide and account-specific values. |
| Standard SIP likely; obtain exact profile | Telnyx; Vonage SIP Trunking | Proceed after confirming country availability, signaling/media ranges, authentication mode, and caller-ID policy. |
| Enterprise SIP likely; sales/engineering validation required | JioBusiness SIP Trunk; Airtel Business SIP Trunk; BSNL Enterprise SIP Trunk | Request technical handoff sheet and written confirmation of customer-owned Asterisk/PJSIP support. |
| Conditional / not proven as a PBX trunk | BSNL Wings consumer/app service | Do not assume consumer SIP credentials authorize Asterisk, automation, or call-center use. Confirm contract/technical support. |
| Unknown until product is identified | Tata Tele Business Services; Bandwidth; DIDWW product/region variants | Ask for a named SIP-trunk product and current interoperability guide; no compatibility verdict from marketing pages alone. |
| Not a drop-in trunk if API/webhook only | Any provider offer that exposes only REST voice APIs or hosted app workflows without SIP interconnect | Would require an API integration or a SIP gateway/product add-on, not just a PJSIP trunk stanza. |

# 8. Provider onboarding questionnaire

Send this list to each carrier’s technical/sales team before implementation:

- Do you support customer-owned Asterisk with chan_pjsip? Which Asterisk versions have been tested?

- Is the ordered product an enterprise SIP trunk, a hosted PBX, a SIP app account, or a programmable voice API?

- Is authentication by REGISTER, SIP Digest on INVITE, source-IP ACL, FQDN, or a combination?

- What exact registrar, termination FQDN, outbound proxy, realm, username format, and contact user are required?

- What are your signaling IP ranges, media IP ranges, signaling ports, RTP port ranges, and supported transports (UDP/TCP/TLS)?

- Is SRTP mandatory or optional? Which TLS versions and certificate rules apply?

- Which codecs and DTMF modes are supported (RFC 4733 telephone-event, in-band, SIP INFO)?

- How must numbers be formatted (E.164 with +, national format, trunk prefix)? Which caller IDs are authorized?

- How are inbound DIDs delivered and identified? Which SIP headers carry the dialed number (Request-URI, To, Diversion, P-Asserted-Identity)?

- What are the concurrent call, calls-per-second, rate, geographic, emergency-call, and premium-number limits?

- What are the failover targets, keepalive/OPTIONS limits, maintenance windows, and support escalation process?

- For India: what are the applicable KYC, numbering, TRAI/DoT, telemarketing/DLT or 140/1600-series requirements for this use case?

# 9. Testing checklist before production

- Confirm registration state only if the product requires registration; otherwise verify IP ACL and SIP OPTIONS behavior.

- Test outbound calls to permitted mobile, landline, and international destinations; verify caller ID and number format.

- Test inbound calls to every DID and check the dialed number as received by Asterisk.

- Capture SIP signaling with pjsip set logger on during a controlled test; redact credentials and personal data before sharing logs.

- Check RTP in both directions, codec negotiation, DTMF, early media, ringback, hold, transfer, hangup cause and CDR fields.

- Test simultaneous calls at expected load and confirm provider concurrency/CPS limits.

- Test failure behavior for unavailable primary URI, lost connectivity, DNS changes, and failover trunk.

- Apply firewall allowlists, strong credentials, outbound route restrictions, fraud/spend limits, alerting, and emergency-call policy.

- Do not expose SIP 5060/5061 and the RTP range globally without a documented security design; use ACLs and an SBC/VPN where appropriate.

# 10. Source directory

Asterisk — Outbound registrations: https://docs.asterisk.org/Configuration/Channel-Drivers/SIP/Configuring-res_pjsip/Configuring-Outbound-Registrations/

Asterisk — PJSIP configuration wizard: https://docs.asterisk.org/Configuration/Channel-Drivers/SIP/Configuring-res_pjsip/PJSIP-Configuration-Wizard/

Asterisk — Secure calling / TLS: https://docs.asterisk.org/Deployment/Secure-Calling/Secure-Calling-Tutorial/

Twilio — Elastic SIP Trunking: https://www.twilio.com/docs/sip-trunking

Twilio — SIP trunk configuration guides: https://www.twilio.com/docs/sip-trunking/sample-configuration

Plivo — SIP trunking quickstart: https://docs.plivo.com/docs/sip-trunking

Plivo — Asterisk interconnection guide: https://docs.plivo.com/docs/sip-trunking/interconnection-guides/asterisk

Plivo — Technical specifications: https://docs.plivo.com/docs/sip-trunking/concepts/technical-specifications

Telnyx — SIP trunk setup: https://telnyx.com/resources/how-to-setup-sip-trunk

Telnyx — SIP endpoint technical details: https://sip.telnyx.com/

Vonage — SIP technical details: https://developer.vonage.com/en/sip/technical-details

Vonage — SIP dashboard setup: https://developer.vonage.com/en/sip/sip-dashboard

JioBusiness — SIP Trunk: https://www.jio.com/business/services/voice-and-collaboration/sip-trunk/

Airtel Business — SIP Trunk: https://www.airtel.in/b2b/sip-trunk

BSNL NGN portal: https://ngn.bsnl.co.in/

BSNL — SIP configuration help: https://ngn.bsnl.co.in/index.php?r=site/confighelp

BSNL Enterprise — SIP application/security form: https://www.calcutta.bsnl.co.in/forms/EB_Form_SIP.pdf

Tata Tele Business Services: https://www.tatatelebusiness.com/

Bandwidth: https://www.bandwidth.com/

DIDWW: https://www.didww.com/

# 11. Practical recommendation for the provider registry

Store provider configuration as structured capability data rather than hardcoding one model: auth_mode (registration / digest / IP ACL / mixed), registrar_uri, termination_uri, outbound_proxy, username, secret reference, source IP allowlist, transport, signaling port, media ranges, codecs, DTMF mode, caller-ID rules, number normalization, inbound DID mapping strategy, concurrency/CPS, failover list, and country/compliance notes. Keep secrets in a secret manager and never in ordinary logs.

Suggested onboarding status values: discovered → sales contacted → technical sheet received → credentials/provisioning complete → lab test passed → production approved. Mark unsupported or unverified fields as unknown rather than guessing. This prevents a generic “SIP provider” label from hiding important incompatibilities.